## # Exploring Router and VLAN security, setting up access lists using Cisco Packet tracer.

- Configure the router and apply ACL rules as follows:-

- 192.168.1.2 deny

- Permit any

- Configure the router and apply ACL rules as follows:-

- Deny the host 192.168.1.1 with the network 192.168.2.0

- Deny the host 192.168.1.2 with the network 192.168.2.0

- Deny the host 192.168.3.0 with the network 192.168.2.0

- Permit all the remaining traffic

| Device Name | Interface Name | IP Address |
| --- | --- | --- |
| Router 0 | Serial 0/0/0 | 202.70.91.253/30 |
|  | Serial 0/0/1 | 202.70.91.249/30 |
|  | Gig 0/0 | 202.70.91.1/28 |
| Router 1 | Serial 0/0/0 | 202.70.91.254/30 |
|  | Serial 0/0/1 | 202.70.91.241/30 |
|  | Gig 0/0 | 202.70.91.17/25 |
| Router 2 | Serial 0/0/0 | 202.70.91.250/30 |
|  | Serial 0/0/1 | 202.70.91.242/30 |
|  | Gig 0/0 | 202.70.91.145/26 |
|  | Gig 0/1 | 202.70.91.209/28 |
| Server 0-3 | Server port | 202.70.91.2-4/28 |
| Laptop 0 | Client port | 202.70.91.5/28 |
| PC3 | Client port | 202.70.91.18/25 |
| Laptop 2-3 | Client port | 202.70.91.19-20/25 |
| PC1 | Client port | 202.70.91.146/26 |
| PC2 | Client port | 202.70.91.147/26 |
| Laptop-1 | Client port | 202.70.91.148/26 |
| PC4 | Client port | 202.70.91.210/29 |
| Laptop-4 | Client port | 202.70.91.211/29 |
| PC5 | Client port | 202.70.91.212/29 |

| On Router-0, create the following ACL Enable. vbbbbb vb conf ter ip access-list standard 10 deny 202.70.91.16 0.0.0.127 deny 202.70.91.208 0.0.0.7 permit any ctrl+Z wr | On Router-1, create the following ACL enable conf ter ip access-list standard complex deny host 202.70.91.150 permit any ctrl+z wr |
| --- | --- |
| On Router-0, create the following ACL | After creating those ACLs, now test the connectivity as |
| enable | per the requirements (the ACL has not yet been applied) |
| conf ter |  |
| ip access-list standard 20 | Ping test from pc3 to server 0 |
| deny 202.70.91.0 0.0.0.15 | PC>ping 202.70.91.2 |
| deny 202.70.91.16 0.0.0.127 | Pinging 202.70.91.2 with 32 bytes of data: |
| deny 202.70.91.128 0.0.0.63 | Reply from 202.70.91.2: bytes=32 time=10ms TTL=126 |
| permit any | Reply from 202.70.91.2: bytes=32 time=1ms TTL=126 |
| exit | Reply from 202.70.91.2: bytes=32 time=1ms TTL=126 |
| ip access-list standard cimish | Reply from 202.70.91.2: bytes=32 time=1ms TTL=12 |
| deny host 202.70.91.146 |  |
| deny 202.70.91.0 0.0.0.127 |  |

| permit any ctrl+z wr |  |
| --- | --- |
| Ping test from pc1 to server0 PC>ping 202.70.91.2 Pinging 202.70.91.2 with 32 bytes of data: Reply from 202.70.91.2: bytes=32 time=2ms TTL=126 Reply from 202.70.91.2: bytes=32 time=1ms TTL=126 Reply from 202.70.91.2: bytes=32 time=1ms TTL=126 Reply from 202.70.91.2: bytes=32 time=1ms TTL=126 | Ping test from pc4 to server0 PC>ping 202.70.91.2 Pinging 202.70.91.2 with 32 bytes of data: Reply from 202.70.91.2: bytes=32 time=2ms TTL=126 Reply from 202.70.91.2: bytes=32 time=1ms TTL=126 Reply from 202.70.91.2: bytes=32 time=1ms TTL=126 Reply from 202.70.91.2: bytes=32 time=1ms TTL=126 |
| Ping test from pc3 to pc4 PC>ping 202.70.91.210 Pinging 202.70.91.210 with 32 bytes of data: Reply from 202.70.91.210: bytes=32 time=1ms TTL=126 Reply from 202.70.91.210: bytes=32 time=1ms TTL=126 Reply from 202.70.91.210: bytes=32 time=1ms TTL=126 Reply from 202.70.91.210: bytes=32 time=1ms TTL=126 | Ping test from pc1 to pc3 PC>ping 202.70.91.18 Pinging 202.70.91.18 with 32 bytes of data: Reply from 202.70.91.18: bytes=32 time=2ms TTL=126 Reply from 202.70.91.18: bytes=32 time=1ms TTL=126 Reply from 202.70.91.18: bytes=32 time=2ms TTL=126 Reply from 202.70.91.18: bytes=32 time=1ms TTL=126 |

Now apply the ACL in to the corresponding interfaces with inbound and outbound settings.

On router 0, in configuration mode, issue the following acl

int gig 0/0

ip access-group 10 out

On router-1, in configuration mode, issue the following acl

int gig 0/0

ip access-group complex out

On router-2, in configuration mode, issue the following acl

int gig 0/0

ip access-group cimish in exit

int gig 0/1

ip access-group 20 out ctrl+z

wr